# Unit 302 Mock exam

# Question 6: For a coil of 0.2H inductance, 35 ohms resistance, on 230V 50Hz supply, calculate: XL, Z and Power factor. Draw an impedance triangle to scale.

First, calculate the XL.

Step one write the base formula

Sometimes the Hertz value is not given. Therefore, assume that the Hz is 50. Whereas, in America it is 60 Hz.

Step two swap in the values

Step three

Deduce the answer XL = 62.83Ω

Second, calculate the Z value.

Z is also referred to as Impedance or Total resistance.

Draw the base formula for Z.

Draw the triangle representation as well.

XL is figured out in part one.

R is provided in the question.

Z = 71.92

Thirdly, calculate the power factor.

= PF => R / Z => 35 / 71.92 = 0.486

= Cos-1(0.486) To convert into degrees

= 60.92