Lesson 16

Worksheet (WS)

Unit 2020 : Electrical science

Worksheet 10: Series/parallel resistive circuits

Work sheet (WS)	2

Generic information:	2

WS Question one	3

WS Question two	4

WS Question three	5

WS Question four	5

WS Question five	8

WS Question six	9

WS Question seven	9

WS Question eight	10

WS Question nine	11

Question ten	12

# Work sheet (WS)

# Generic information:

In a series circuit we add all of the resistance/resistors.

Whereas, in a Parallel circuit we use the formula.

# WS Question one

|  | R1 | R2 | R3 | R4 | Rt |
| --- | --- | --- | --- | --- | --- |
| a | 7 Ω | 10 Ω | 8 Ω | 15 Ω | 40 Ω |
| b | 7 Ω | 16 Ω | 8 Ω | 19 Ω | 50 Ω |
| c | 1.5 Ω | 5.6 Ω | 8.2 Ω | 7.3 Ω | 22.6Ω |
| d | 0.03 Ω | 0.105 Ω | 1.06 Ω | 2.007 Ω | 3.202Ω |
| e | 15 MΩ | 21.3 MΩ | 1.4 MΩ | 5.3 MΩ | 43MΩ |
| f | 15 mΩ | 83 mΩ | 26 mΩ | 9 mΩ | 133MΩ |
| g | 200 KΩ | 500 KΩ | 10 kΩ | 1.2 MΩ | 1.91MΩ |

Question one working out

# WS Question two

2 a) working out

In a series circuit the current is constant. Whereas, in the series circuit the voltage changes. We have to deduce the value of R4. For the resistances we have:

R1 = 5 ohms,

R2 = 23 ohms,

R3 = 10 ohms,

R4 = ?

The Ohms law triangle.

110 V / 2A = 55 ohms

55 Ohms is the total resistance for the entire circuit.

R1 (5 ohms) + R2 (23 ohms) + R3 (10 ohms) = 38 ohms

Hence, R4 = 17 ohms.

Because, 38 + 17 = 55 ohms.

2 b) working out

Voltage drop in a resistor = Current x resistance of resistor

| Resistor | Current | Resistance per resistor | Voltage drop per resistor |
| --- | --- | --- | --- |
| R1 | 2 | 5Ω | 10 |
| R2 | 2 | 23Ω | 46 |
| R3 | 2 | 10Ω | 20 |
| R4 | 2 | 17Ω | 34 |
|  |  | Voltage drop total | 110V |

The way to figure out if you have calculated voltage drop properly. Is that the total voltage drop should equal the voltage of the entire circuit.

# WS Question three

Calculate the current following through each resistor

A)  The current flowing in the resistor

R3 => 2.4 V /0.012 ohms = 200 amps

R2 => 2.4 V /0.015 ohms  = 160 amps

R1 => 2.4 V /0.008 ohms = 300 amps

B) The total current drawn from the supply

Total Current = Total Voltage / Total Resistance

2.4 / (200 + 160 + 300 = 660)

= 0.035 amps

# WS Question four

Question four working out

# WS Question five

Question five working out

Row A) R2 = 80 Ω

Row B) R1 = 16 Ω

Row C) R1 = 200 Ω

Row D) R2 = 40Ω

Row E) R2 = 30Ω

# WS Question six

# WS Question seven

Question seven working out

# WS Question eight

Question eight working out

In a series circuit we add the resistor values together to get the Resistance Total (Rt).

In this case we divide 14.4 ohms by 6 to get the individual resistor values. The answer is 2.4.

# WS Question nine

Question nine working out

# Question ten

Question ten working out