# Site Experience #6.00

Site Experience #6.00	1

Brief	1

Reflective - What did I learn?	1

End to End test	2

The figure of eight test	2

The figure of eight test we conduct the following two tests:	2

For example, if the R1 and RN = 0.40 ohms. Then we expect
R1 and R2  = 0.668 ohms.	2

Comparatively, R1 and R2 should be 1.67 times higher than the R1 and RN wire. This is because 2.5mm2 divide by 1.5mm2 = 1.67.	2

Loop impedance test	2

# Brief

On this day I was based the entire day in Oakham the whole day as I worked on an installation project.

# Reflective - What did I learn?

- Ring circuits were introduced after World War Two because there was a copper shortage.

- Sockets are always on a ring final circuit.

- In the rose ceiling circuit we placed all the red wires into the Loop section.

- The cooker is on a radial circuit.

- Lights are on a radial circuit.

# End to End test

Is a continuity test which ensures that the wire is connected the whole way around.

# The figure of eight test

# The figure of eight test we conduct the following two tests:

R1 and R2

R1 and RN.

# For example, if the R1 and RN = 0.40 ohms. Then we expect
R1 and R2  = 0.668 ohms.

# Comparatively, R1 and R2 should be 1.67 times higher than the R1 and RN wire. This is because 2.5mm2 divided by 1.5mm2 = 1.67.

# Loop impedance test

He performed a test in which no modifications were made to the fuse board. On the tester there are two sections: Live testing and Dead testing.

He plugged the chords into Live, Earth and Neutral. Then on the opposite end he plugged a three pin plug into the sockets.

He mentioned that this is a Live test and checks the Earth Fault Loop impedance.

On the ring circuit there were for plugs the readings were as follows:

First socket: 0.30

Second socket: 0.32

third socket:0.30

Fourth socket: 0.30

# Photographic content

Connecting a humidistat time fan