Question on Conduit calculations:	1

# Question: calculate the tabulated current

A 5.5A load has a conductor protected by a 6A mcb and has the following correction factors applied to it:

Ca = 0.94 & Cg = 0.6

Calculate the “tabulated current” required.

When it says “tabulated current” it only wants you to calculate the up to the It

# Question on Conduit calculations:

The following cables are to be drawn into a straight 2 metre length of conduit:

- 2 of solid core 1.5mm2

- 4 of solid core 2.5mm2

- 4 of stranded 4mm2

Calculate the conduit size required to accommodate these cables?

Workings out:

- The wires core. Which means the copper wires inside of the insulation can be solid or grouped together as strands. Stranded wires provide greater flexibility than solid ones.

- Sometimes the question will specify the length E.g. 2 metres. Otherwise it will specify Short.

- For cable factors in short straight runs use table E1.

Step one - use table E1 to get the cable factor

Step two - create a table to calculate the total cable factor

| No. of cables | size | Cable factor | (No. of cables X Cable factor) |
| --- | --- | --- | --- |
| 2 | 1.5 | 27 | 54 |
| 4 | 2.5 | 39 | 156 |
| 4 | 4 | 58 | 232 |
|  |  | Total Cable factor | 445 |

Use the total cable factor to look-up the conduit factor.

Go to Table E4. As a side note I know that the question clearly states “Straight 2 metre length” which is a short straight run. Therefore, why would we use Table E4 which is for long runs?

However, the left corner stipulates “Covered by Tables E1 and E2”.

But, it cannot be Table E1 because we used it for the cable factor calculations hence it must be Table E2.

We use the total cable factor 445 and round it up to the nearest value in the Conduit factor column which is 460. We can look-up this value in the conduit diameter (mm) column so that value is 20(mm).