Sunday 7th September 2025

Lesson 18

IWA

Class room - Worksheet walkthrough	2

Question 1	2

Question 2:	3

Question 3	4

Question 4	4

Question 5	5

Question 6	5

Question 7	6

Question 8	6

Question 9	7

Question 10	8

Question 11	9

Question 12	9

Question 13	10

Question 14	10

Question 15	11

Question 16	11

Question 17	12

Question 18	12

Question 19	12

Question 20	13

Question 21	14

Question 22	14

Question 23	16

Question 24	18

Question 25	18

Question 26	19

Question 27	20

Question 28	21

Question 29	22

Question 29 working out	22

Question 30	22

Question 31	22

Question 32	23

Question 33	23

Question 34	23

Question 35	24

Question 36	24

Question 37	24

Question 38	24

Question 39	25

Question 40	25

Classroom theory	25

# Class room - Worksheet walkthrough

5357 - 003 Electrical Scientific Principles and Technologies /

2365-602 Principles of Electrical Science

# Question 1

# Question 2:

Question 2 working out:

Foundation:

The formula a² + b² = c² is known as the Pythagorean theorem, which states that in a right-angled triangle, the square of the hypotenuse (the longest side) is equal to the sum of the squares of the other two sides (the legs).

This means that if you square the lengths of the two shorter sides (a and b) and add them together, you get the same value as squaring the length of the longest side (c).

Original source: google

Answer is 6 cm

C = The hypotenuse which is the longest side of the triangle.

a = 8 cm | c = 10 cm | b = ?

- Following, the rule we know that performing the addition:

- a2 + b2 = c2

- a2 + b2 equates to c2

The question already gave us the value for c to be 10. So c2 = 100.

Therefore, a2 + b2 has to equal 100.

We already have a value for a to be 8 cm. So a = 8 cm.

82 = 64. Which is 36 away from 100.  The square root of 36 is 6. Therefore the answer for b = 6 cm.

# Question 3

Question 3 working out

The answer is C.

Perform the multiplication 3 m x 8 m x 2.4 m = 57.6m2

# Question 4

Question 4 working out

The answer is Kelvin

# Question 5

Question 5 working out

The answer is B frequency

# Question 6

Question 6 working out

The answer is C

# Question 7

Question 7 working out

The answer is A

# Question 8

Question 8 working out

The answer is A) weight = mass x gravity

# Question 9

Question 9 working out

The answer is B) A lever

# Question 10

Question 10 working out

There are 4 pulleys (represented by four circles)

The weight is 500 kg.

The formula is: Force = Mass x gravity

4905 = 500 kg x 9.81

# Question 11

Question 11 working out

The answer is A) Potential energy

# Question 12

Question 12 working out

The answer is B => 2.1 / 0.85 = 2.47

# Question 13

Question 13 working out

# Question 14

Question 14 working out

5 / 0.80 = 6.25 kW

# Question 15

Question 15 working out

The answer is A) Protons

# Question 16

Question 16 working out

The correct answer is D) Copper.

Gold is also a conductor of electricity.

# Question 17

The correct answer is A) 0.2 ohms.

When you half the distance you half the resistance.

# Question 18

Question 18 working out

# Question 19

Question 19 working out

The circuit shown is a Parallel circuit.

The answer is D.

In a parallel circuit; the voltage is constant, the current differentiates.

In a series circuit; the current is the same and the voltage changes based on the resistors.

# Question 20

Question 20 working out

The answer is A) The voltage varies across the resistors but the current is constant.

# Question 21

Question 21 working out

VIR formula triangle.

V = I x R.

Therefore, 5 A (the current is constant in a series) x 20 ohms of resistance -

# Question 22

Question 22 working out

The answer is B) 50 V.

This circuit is a series circuit.

In a series circuit the current is constant and the voltage changes around the resistors.

The total voltage is given as 200 V.

We can calculate the total resistance as (20Ω + 30Ω + 30Ω) = 80Ω.

We can then calculate the total current for the series circuit = 2.5 A.

I = V / R  => 2.5 A = 200 V / 80Ω

The current of 2.5 A is constant in a series circuit.

Then we do Ohms law around each resistor.

V1 = 2.5 A x 20Ω => 50 V

V2 = 2.5 A x 30Ω = > 75 V

V3 = 2.5 A x 30Ω = > 75 V

# Question 23

Question 23 working out

VT = 100 V

IT = ?

RT = 90Ω (we do not add up the resistor/resistance values as we would do in a series circuit)

1/rt = 1/30 + 1/30 + 1/30

0.1 = 0.0333 + 0.0333 + 0.0333

0.1 => 10Ω (multiply by 100)

IT = 100 V / 10Ω

IT = 10 amps

In a parallel circuit; the voltage is constant and the current differentiates.

# Question 24

Question 24 working out

The answers is D

The current is constant in a series circuit and the voltage changes around the resistors.

I = Voltage / Resistance.

200V / 50 ohms Ω = 40 amps

# Question 25

Question 25 working out

The answer is C) 10 Ω

The questions: “What would be the total circuit resistance if a parallel circuit contained five resistors all having a 50Ω?”

# Question 26

Question 26 working out

The answer is A) Series.

In a series circuit you can calculate the total resistance by adding all of the individual resistors together.

# Question 27

Question 27 working out

In a parallel circuit the voltage is constant. The current differentiates.

Whenever power is mentioned the formula to use is PIV. P = I x V.

The question provides us with the voltage and the information to calculate the total resistance. However, we need to use the VIR formula triangle first to get the I (current) in order to calculate the power.

V = 100V

I = ?

R =>

1/30 + 1/30 + 1/30 = 3/30.

3/30 simplified = 1/10 => 0.1.

0.1 x 100 = 10Ω.

100V / 10Ω = 10 amps

V = 100V

I = 10 amps

R = 10Ω

Now we have the current and the voltage so we can deduce the power. Using the power formula triangle.

Power = Current x Voltage

1kW  = 10 amps x 100V

# Question 28

Question 28 working out

The answer is B

# Question 29

# Question 29 working out

The answer is A

# Question 30

The answer is B

# Question 31

Question 31 working out

The answer is B

# Question 32

Question 32 working out

# Question 33

# Question 34

# Question 35

# Question 36

# Question 37

# Question 38

# Question 39

# Question 40

# Classroom theory

Mass never changes and stays the same.

Weight changes due to the impact of gravity. A person would have a different weight on earth compared to outer space.