# Lesson #5.00 2365 - Level 3 City and Guilds

Question one: Determine the reactance when an inductance of 40 mH is connected to a 230 V, 50 Hz supply?	1

Question two: Determine the reactance when a 120 µF capacitor is connected to a 230 volt, 50 hertz supply.	3

# Question one: Determine the reactance when an inductance of 40 mH is connected to a 230 V, 50 Hz supply?

- Write the formula

- Swap in the values for the formula

The answer to the question is 12.57Ω. For the question we want to find the resistance within the coils that is the definition of XL.

Question one: Additional steps to calculate current through the coils.

To calculate the current through the inductor/the coils. The steps are:

- Write the detailed formula for Ohm’s Law

- Write the detailed formula for Ohm’s Law. Swap in the values.

Remember, the voltage was provided in the question as 230 V and the XL was calculated in the first part of the answer as 12.57Ω.

Therefore, to calculate the Current/I we do 230V/12.57Ω.

# Question two: Determine the reactance when a 120 µF capacitor is connected to a 230 volt, 50 hertz supply.

- Display the formula

- Swap in the values within the formula.

- Additional steps to determine the current in amps.

- The phases 120 degrees apart from one another.

- L1, L2 and L3

- Do not need the neutral in a balance system. There is no connection for the neutral.

- In an unbalanced system you need a neutral. The neutral is there to take the current away.

# Example one:

Star connection

A three-phase, four wire star connected transformer has a line voltage of 520 volts and supplies a line current of 25 amps. Calculate.

- The phase voltage of the transformer

- The phase current in reach winding

- The phase voltage of the transformer

- The phase current in each winding

The answer for the question is relatively simple.

In a star relationship IL equals IP.

= 25 amps.

Delta connection

Example two - A three-phase balanced delta connected resistive load is supplied from a transformer at a line voltage of 400 volts and draws a line current of 13.86 amps. Calculate:

- A phase voltage

- The phase current

- The resistance of the load

A The Phase voltage

- In a phase voltage is equal to the line voltage.

- We know that the 400 Volts is the voltage of the line.

B The phase current

Rearrange the current to find Ip

=>

C The resistance of the load

The formula follows that of Ohms law.

Therefore: